Simple Methods to Modify GHG Inventories

[It should be clear that the document scope is materials mgmt/waste and consumption and so a shorter title should work]

Shortcomings with the current methodology for estimating emissions from the waste sector in GHG inventories are being addressed in several local and state inventories by two general approaches.  One involves adjustments to the assumptions in the current methodology for waste GHG emissions.  The other approach goes beyond a waste focus and involves several approaches to present emissions associated with materials consumption.  Outlined below are these two general, “simple” methods for modifying the GHG inventories conducted by communities.

[Questions on terminology:  Is materials management or materials consumption a better term here?  Should we use “systems approach” more than “material management”?]

Modifying Waste Sector GHG Inventory Methodology

[Comments/Questions:  We think that we don’t need to expound on the first bullet as we believe this is standard practice in the ICLEI model.  Is this true?  Are the issues around gas capture already covered somewhere else?  Is the waste to energy issue significant enough to include.  Perhaps this section should only address the simple methods for presenting materials management and consumption emissions rather than also address modification to how waste is treated in the sector inventory.]

Original outline

- Waste disposed by the community, not in the community (although this is probably already standard practice, so may not need to expound on it)

- Gas capture rates (what would we recommend as a default, or defaults for different conditions)?

- Treatment of energy recovery from waste (avoid double-counting)

Simple Methods to Present GHG Emissions from Materials Consumption

Qualitative Method

Typically the results of GHG inventories are presented in Climate Action Plans or in stand alone documents.  The “qualitative method” involves adding to these documents a narrative discussion on the nature and significance of GHG emissions associated with the consumption of materials by a community.  This qualitative presentation is used to give visibility to these emissions, which primarily fall outside the scope of the typical GHG inventory.  A good example of this approach can be found in the Climate Action Plan for the City of Portland and Multnomah County (see http://www.portlandonline.com/bps/index.cfm?c=49989&a=268612).  This approach and presentation can easily be adapted for use in any community GHG Inventory and Climate Action Plan.

A traditional sector based GHG inventory was conducted for the Portland area Climate Action Plan, which identified local energy use (residential, commercial and industrial) and transportation emissions as the primary sources of community GHG.  The Plan is noteworthy, however, for also acknowledging the role of consumption of goods (and related emissions that are traditionally excluded from community GHG inventories). On pages 21 and 22, the Plan presents the EPA "sector" and "system" based views of emissions to make the following point:

"Taken together, the traditional and complementary approaches to inventorying emissions offer insight into the underlying causes of — and therefore the opportunities to reduce — carbon emissions. Both approaches are needed because the businesses and industries located in Multnomah County produce different kinds and quantities of goods than what local residents consume. Examining carbon emissions through both methods therefore provides a more complete picture of the total emissions for which Portland and Multnomah County bear some responsibility.”

Although Portland has not calculated or estimated its own emissions resulting from consumption of goods, the inventory component of the Plan makes a compelling case for acknowledging and addressing these emissions.  Materials management and consumption related actions were then incorporated into the Plan.  Typical of more progressive Climate Action Plans, the Portland Plan includes a waste diversion goal – 90% diversion by 2030 – to increase recycling and composting.  It also takes the innovative approach of including a goal to reduce solid waste generation – 25% reduction by 2030.  This action is specifically intended to measure a reduction in material consumption by the community as a result of reduce/reuse strategies.  In total, Portland’s consumption and solid waste actions (includes those related to food consumption practices) account for 46% of targeted reductions to achieve their overall goal of 40% reductions in emissions by 2030.

Per Capita Method

The per capita method involves using national per capita GHG emissions derived from EPA’s “systems” inventory.  EPA estimated that U.S. GHG emissions in 2006 were 7,054 MMTCO2E.  This converts to per capita emissions of approximated 23 metric tons.  This is substantially higher than the per capita emissions calculated from many community inventories since these inventories are missing the lifecycle emissions of materials consumption as well as some transportation emissions.  EPA’s systems inventory estimated that 42% of these emissions are associated with the provision of goods and food (collectively are referred to as materials management).  The per capita estimate for these emissions is 10 metric tons.  To account for the lifecycle emissions associated with a community’s materials consumption, the 10 tons per capita emissions for materials can be multiplied by a community’s population to get a rough baseline of these emissions.  This total can be compared with emissions from the sector inventory to give a more complete picture of the emissions that a community has some influence over (e.g., through their consumption patterns).

In making this comparison it should be recognized that the industrial sector emissions estimated by the traditional inventory are a component of the materials consumption emissions derived from the national data.  The main difference is that industrial sector emissions in a community are not correlated to materials consumption by the public in a community and may be outside the influence local governments.

The Portland metropolitan area government is currently developing this approach.  Aside from using the materials emissions estimates from EPA’s systems inventory, they will also use the data from transportation and energy, and they plan to make slight adjustments for local conditions.  This per capita can be used as an augmentation to the qualitative method described above.

Carbon Footprint Method

The carbon footprint method involves complimenting the traditional inventory with a carbon footprint calculation to provide an estimate of the lifecycle emissions associated with consumption of goods and services.  Lifecycle emissions associated with materials consumption are determined by a carbon footprint calculation.

One example of this approach in a Climate Action Plan can be found in the plan developed for the University of California, Berkeley campus (ADD Link).  Lifecycle emissions associated with consumption of goods and services by the University were calculated using an Economic Input-Output Lifecycle Assessment (EIO-LCA) calculator.  This approach more than doubled the GHG emissions from campus activities as compared to the traditional inventory.  This information is being used as a factor in procurement decisions by the University.  The State of California has also funded UC Berkeley to use this same approach to develop an EIO-LCA calculator for community and individual carbon footprints.  The calculator can be found at the State of California’s web site CoolCalifornia.org (ADD link).  This calculator could be used in community inventories following the approach taken by UC Berkeley.  Such an approach would help educate the public on the GHG impacts of their consumption and could be used by cities to inform their procurement practices.  [Are there other calculators using the EIO approach that should be referenced?  Check how widely the current calculator is for use by communities on the west coast?  Is there a fee?]

Waste Composition Method

[Based on Fort Collins model.  Include here or under advanced methods.]